● Purpose : Simulation of heart valve model .

● Problem : Comsol don't recognize self-made valve for simulation.

● Present setup

- model wizard - 3D – Laminar flow – time dependent

1. Process

- Heart and valve models are implemented separately and then combined.

- Uncheck simplify mesh of CAD file and import.

- If set to Form a union, an error occurs, set to Form a assembly.

- Set the imported model to detect boundaries as boundary partitioning of mesh part.

- Modify the detect angle at detect boundaries to set the boundaries of the mesh.

- meterials setting : density = 1055 kg/m^3, dynamic viscosity = 0.0035 Pa·s

- boundary condition setting : Pulmonary vein=inlet, Aorta=outlet, etc=wall

- mesh setting : Separately set the mesh size of the heart and valve

2. Problem

- Compute after completing materials, boundary conditions, and mesh settings, it will appear as follows.

- Currently used model is MV(mitral valve) close, AV(aortic valve) open.

- The MV is in close condition, but the stream line is created by passing through the blocked valve.

- Streamline should be formed only in the open part of AV, but the whole is created.

[AV]			        [MV]

MV

AV

MV

AV

AV

MV

[streamline]

3. Solution investigation

- If interior wall is set, valves in the model are recognized and streamlines are expected to not flow.

- But can't set interior wall.